India Started License Request

- When requesting a license for a PBX for the first time, go to the License → Edit section from the Xymphony interface to access the license request screen.
By default, the initial package values are automatically preloaded on this screen.

The contents of the initial package are as follows:

  - 4 IVR Channels

  - 8 VoIP Channels

  - 4 XSIP

  - 1 Xcom

  - 4 Trunk Lines

  - 40 Analog Subs

  - 2 Auto Recording

- At the bottom of the screen, you can select India (IN) from the IN / Custom button to submit a license request using this package.
If desired, you can increase or decrease the default values according to your requirements.

- If you wish to request a license with a different configuration instead of the default package, select Custom from the Selection Template section at the bottom of the screen and click OK.
Then, return to the License → Edit section and enter your desired configuration to create the request.

- If a license is already installed on the PBX, the Selection Template option will no longer appear, and the currently installed license details will be displayed.
In this case, you can simply enter the additional values you want to request.